| Equipment Name: | Cooling Pad |
| --- | --- |
| Required for: | Placing slides for testing |
| Equipment ID: | CP01 |
| Document No: | CPPQ01 |

| Sr. No | Description | Page No. |
| --- | --- | --- |
| 1 | Protocol Approval | 2 |
| 2 | Introduction | 2 |
| 3 | Objective | 2 |
| 4 | Responsibilities | 4 |
| 5 | Qualification Tests | 5 |
| 6 | Non-conformance & Corrective Action | 9 |
| 7 | Summary and Conclusion | 10 |
| 8 | Report Approval | 10 |
| 9 | Revision History | 10 |

# 1. PROTOCOL APPROVAL:

| Details | Designation | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Checked By |  |  |  |
| Approved By |  |  |  |

2. INTRODUCTION:

# This Performance Qualification (PQ) report documents the final phase of qualification for the Peltier Cooling Pad. Having successfully completed Installation Qualification (IQ) and Operational Qualification (OQ), this phase focuses on evaluating the system's ability to maintain specific thermal baselines under active biological/chemical sample loads or simulated operational stresses.

3. OBJECTIVE:

- The device achieves the required cooling rate and reaches the target temperature within an acceptable time when cooling a representative thermal load.

- The stepwise temperature-time profile is executed accurately, with correct transitions, timing, and temperature holding tolerances.

- The system demonstrates repeatability across multiple independent runs.

- The device maintains stable long-term temperature control over extended operating periods without performance degradation.

4. RESPONSIBILITIES:

- Validation Engineering Department: Responsible for drafting the PQ protocol, setting up external mapping sensors, executing the continuous-run tests, and compiling final data charts.

- Lead Project Engineer: Responsible for adjusting PID coefficients if required for load tuning, maintaining software configuration, assisting with technical diagnostics and filling water in container.

- Clinical/Laboratory Operations: Responsible for providing standard sample arrays, simulating routine user actions, and executing Standard Operating Procedures.

- Quality Assurance Department: Responsible for auditing raw data logs, reviewing non-conformances, and granting final operational clearance.

5. QUALIFICATIONS:

- Cooling Capacity Test: Test is performed to verify the device can cool the representative thermal load from ambient temperature to a set target within a reasonable time frame.

- Profile Execution: Each time and temperature profile step is executed correctly while the device is loaded. All the profiles must run without any interruption.

- Run Repeatability: It is confirmed that the cooling performance and profile execution are consistent and repeatable across multiple independent runs.

- Stability: Properly evaluated the device's mechanical and thermal stability during continuous operation under load.

6. NON-CONFORMANCE:

Any deviation, failure, or unexpected observation during the PQ execution shall be documented in this section.

| Date | Non-Conformance | Sign |
| --- | --- | --- |

7. SUMMARY & CONCLUSION:

|  | Cooling Capacity Test |
| --- | --- |
|  | Stepwise Profile Execution Under Load |
|  | Run‑to‑Run Repeatability |
|  | Long‑Term Stability |
|  | Web Interface / Communication Reliability |

Based on these results, the system is declared fully validated and is certified as fit for its intended clinical, research, and diagnostic applications.

8.PERFORMANCE QUALIFICATION REPORT APPROVAL:

| Details | Designation | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |

# 9. REVISION HISTORY:

| Revision No. | Change Control No. | Reason for revision | Changes Incorporated |
| --- | --- | --- | --- |